1. struct napi_struct — Core NAPI Structure

This is the central structure for each NAPI instance, usually in a network driver.

Key fields (simplified):

| Field | Description |
| --- | --- |
| struct list_head poll_list | Links NAPI to per-CPU poll list |
| int weight | Budget: max packets to process in poll |
| int state | NAPI state: idle / scheduled / running |
| struct net_device *dev | Associated NIC device |
| void (*poll)(struct napi_struct *, int) | Pointer to driver poll function |
| unsigned int rx_pending | Optional: RX descriptor counter |
| void *driver_data | Pointer to driver private data |

Explanation:

- Each NIC RX queue usually has one NAPI instance

- Poll function pointer → called in softirq context

- weight ensures fair CPU allocation

2. Poll Function (napi->poll)

Responsibilities:

- Process RX descriptors

- Convert descriptors into sk_buff

- Deliver packets to network stack

- Clean TX descriptors (optional)

- Track work_done and check against budget

SoftIRQ Context:

- Runs inside NET_RX_SOFTIRQ

- Cannot sleep

- Must obey budget

3. Poll List (napi_struct->poll_list)

- Each CPU has a per-CPU poll list

- Links all scheduled NAPI instances

- SoftIRQ iterates all poll_list entries sequentially

- Ensures fair CPU time for multiple NAPIs on same CPU

4. NAPI State Machine

| State | Meaning |
| --- | --- |
| IDLE | No packets to process; RX interrupts enabled |
| SCHEDULED | NAPI scheduled in poll list, waiting for softirq |
| RUNNING | Poll function executing in softirq context |

Transitions:

- IDLE → SCHEDULED → via ISR + __napi_schedule()

- SCHEDULED → RUNNING → softirq calls poll()

- RUNNING → IDLE → poll completes (work_done < budget) + napi_complete_done()

- RUNNING → SCHEDULED → poll returns work_done == budget, more packets pending

5. ISR / Interrupt Handler Interaction

NAPI Components in ISR:

- napi_schedule_prep(&napi) → checks if NAPI already scheduled

- __napi_schedule(&napi) → marks NAPI scheduled, raises NET_RX_SOFTIRQ

- disable_irq_nosync() → prevents interrupt storm

Notes:

- ISR does minimal work

- Polling is deferred to softirq context

6. Budget / Weight Component

- napi->weight → maximum packets processed per poll

- Ensures fair CPU allocation among multiple NAPIs

- If packets > weight → poll continues next softirq cycle

- If packets < weight → NAPI completes → interrupts re-enabled → fairness preserved

7. SoftIRQ Integration

- NAPI relies on NET_RX_SOFTIRQ for execution

- SoftIRQ iterates per-CPU poll_list

- Calls poll function of each NAPI instance

- Manages scheduling and fairness between NAPIs

8. RX Descriptor Handling

Driver-level component:

- NAPI is closely tied to RX rings

- Poll function reads descriptors:

  - Checks status

  - Unmaps DMA buffer

  - Builds sk_buff

  - Passes packet to network stack (napi_gro_receive())

9. TX Cleanup Component (Optional)

- Many drivers do TX completion in poll

- Reduces CPU wakeups

- Reclaims sk_buff memory for TX

- Done in same poll loop for efficiency

10. Interrupt Enable/Disable Component

Driver logic tied to NAPI:

- RX IRQ disabled when NAPI scheduled

- Re-enabled in napi_complete_done() only if poll finishes early (work_done < budget)

- Prevents interrupt storm and improves throughput under heavy load

11. ksoftirqd Fallback (Optional Component)

- If softirq execution exceeds CPU time:

  - Kernel defers processing to ksoftirqd thread

- Runs in process context, can sleep

- Ensures system stability under extreme traffic

- NAPI continues polling there if necessary

12. Multi-Queue / Multi-NIC Components

- Each NIC RX queue has one NAPI instance

- SoftIRQ iterates poll list sequentially:

  - Poll NIC1 → poll NIC2 → …

- Budget ensures fair CPU allocation among NICs on same CPU

- Each CPU handles its own NAPI poll list independently

13. NAPI Component Summary Table

| Component | Role | Key API / Function |
| --- | --- | --- |
| struct napi_struct | Holds NAPI state | poll, weight, dev |
| Poll function | Process RX/TX descriptors | napi_poll(napi, budget) |
| Poll list | Schedule NAPIs on CPU | poll_list |
| State machine | IDLE, SCHEDULED, RUNNING | napi_complete_done() |
| Budget/Weight | Limit CPU per poll | napi->weight |
| ISR interaction | Schedule NAPI, disable RX IRQ | napi_schedule_prep(), __napi_schedule() |
| SoftIRQ | Calls poll in deferred context | NET_RX_SOFTIRQ |
| RX descriptors | DMA + packet delivery | dma_unmap(), build_skb(), napi_gro_receive() |
| TX cleanup | Optional TX reclaim | driver-specific |
| IRQ enable/disable | Prevent interrupt storm | enable_irq(), disable_irq_nosync() |
| ksoftirqd | CPU fallback | ksoftirqd/<cpu> |

[NIC RX Queue] → [RX IRQ / ISR] → [NAPI scheduled] → [SoftIRQ] → [Poll function] → [Budget check] → [NAPI complete / continue] → [RX IRQ re-enabled]

- Budget = fairness knob

- Multiple NICs → sequential polling in per-CPU poll list

- Poll function = main workhorse

- ISR = lightweight scheduler