Detailed Low-Level Steps: From NIC Interrupt to Packet Delivery in Linux Kernel Networking Stack

This is an in-depth breakdown of the entire packet reception path; from the moment a packet arrives at the NIC to when it is delivered to a user-space application.

Step 1: Packet Arrives at the NIC (Hardware Layer - L1)

What Happens?

A network packet arrives on the physical network interface. The Network Interface Card (NIC) receives the packet and places it in its RX ring buffer (DMA buffer).The NIC raises a hardware interrupt (IRQ) to notify the CPU.

Low-Level APIs / Functions Involved.

NIC Driver Registers an IRQ Handler (During Initialization)

request_irq(nic_irq, my_irq_handler, IRQF_SHARED, "my_nic", dev);

NIC Interrupt Handler (ISR)

static irqreturn_t my_irq_handler(int irq, void *dev_id) {

struct my_eth_dev *dev = dev_id;

disable_interrupts();  // Disable further NIC interrupts

napi_schedule(&dev->napi);  // Schedule NAPI for packet processing

return IRQ_HANDLED;

}

✔ disable_interrupts() → Prevents too many interrupts (for performance).

✔ napi_schedule() → Schedules a softirq instead of processing packets in the ISR (Interrupt Service Routine).

Step 2: SoftIRQ (NET_RX_SOFTIRQ) is Triggered.

What Happens?

The kernel defers packet processing to a SoftIRQ (NET_RX_SOFTIRQ).This allows batch processing of packets instead of handling them one by one in interrupts.

Low-Level APIs / Functions Involved

SoftIRQ Execution (Triggered by ISR or ksoftirqd)

void do_softirq(void) {

if (softirq_pending(cpu)) {

run_softirq();

}

}

SoftIRQ Calls net_rx_action()

void net_rx_action(struct softirq_action *h) {

struct napi_struct *napi;

list_for_each_entry(napi, &sd->poll_list, poll_list) {

int work = napi->poll(napi, budget);

}

}

✔ do_softirq() → Runs pending SoftIRQs.

✔ net_rx_action() → Calls NAPI poll functions to process packets in batches.

Step 3: NAPI Polling Fetches Packets from NIC RX Queue

What Happens?

The NAPI poll function fetches multiple packets from the RX ring buffer.Each packet is processed up to a given budget to prevent CPU starvation.

Low-Level APIs / Functions Involved

NAPI Poll Function

static int my_poll(struct napi_struct *napi, int budget) {

int processed = 0;

while (processed < budget) {

struct sk_buff *skb = fetch_packet_from_nic();

if (!skb) break;  // Stop if no more packets

netif_receive_skb(skb);  // Pass packet to networking stack

processed++;

}

if (processed < budget) napi_complete(napi);

else napi_schedule(napi);

return processed;

}

✔ fetch_packet_from_nic() → Reads packets from the NIC RX buffer (DMA mapped).

✔ netif_receive_skb(skb) → Passes packets to Linux networking stack.

✔ If all packets are processed, napi_complete() re-enables interrupts.

✔ If packets are still pending, napi_schedule() reschedules SoftIRQ.

Step 4: Packet Passed to Networking Stack (L2 - Ethernet)

What Happens?

The Ethernet (L2) header is processed.The protocol type (IPv4, IPv6, ARP, etc.) is checked.

Low-Level APIs / Functions Involved

Pass to L2 Processing

int netif_receive_skb(struct sk_buff *skb) {

return __netif_receive_skb(skb);

}

int __netif_receive_skb(struct sk_buff *skb) {

if (skb->protocol == htons(ETH_P_IP)) {

return ip_rcv(skb);  // Process IPv4 packet

} else if (skb->protocol == htons(ETH_P_IPV6)) {

return ipv6_rcv(skb);  // Process IPv6 packet

}

}

✔ Ethernet header is removed, and the protocol field determines the next step.

✔ If it’s an IP packet, it is passed to ip_rcv().

Step 5: Packet Routed at Network Layer (L3 - IP)

What Happens?

The IP header is validated (checksum, TTL, etc.).The packet is either forwarded or locally delivered.

Low-Level APIs / Functions Involved

IP Packet Processing

int ip_rcv(struct sk_buff *skb) {

if (ip_is_local(skb)) {

return ip_local_deliver(skb);  // Packet for this machine

} else {

return ip_forward(skb);  // Packet should be forwarded

}

}

✔ If this system is the destination, it is passed to ip_local_deliver().

✔ If the packet needs to be routed, it is sent to ip_forward().

Step 6: Packet Processed at Transport Layer (L4 - TCP/UDP)

What Happens?

If it’s a TCP or UDP packet, it is sent to the appropriate socket buffer.

Low-Level APIs / Functions Involved

TCP / UDP Processing

int ip_local_deliver(struct sk_buff *skb) {

if (ip_hdr(skb)->protocol == IPPROTO_TCP) {

return tcp_v4_rcv(skb);

} else if (ip_hdr(skb)->protocol == IPPROTO_UDP) {

return udp_rcv(skb);

}

}

✔ If it’s a TCP packet, it is sent to tcp_v4_rcv().

✔ If it’s a UDP packet, it is sent to udp_rcv().

Step 7: Packet Delivered to User Space Application

What Happens?

The packet is placed in the socket receive queue.The user-space application reads it using recv() or recvfrom().

Low-Level APIs / Functions Involved

Packet Added to Socket Buffer

sk_add_backlog(sk, skb);

User-Space Reads Data

recv(sockfd, buffer, sizeof(buffer), 0);

✔ The application receives the packet via system calls.

✔ Interrupts are disabled early, and NAPI reduces CPU overhead.

✔ Packets are processed in batches in napi_poll().

✔ Linux kernel efficiently moves packets through L2, L3, and L4.

✔ Packets are finally delivered to the user-space application.