🟦 1. What is a Switch?

A switch operates at:

- Layer 2 (Data Link Layer) – MAC addresses

- Some switches also support Layer 3, but mainly switching is L2

✔️ What it does

- Connects devices inside the same network

- Learns MAC addresses

- Forwards frames within LAN

- Creates VLANs for segmentation

✔️ Example

Devices in an office:

PC ↔ Switch ↔ PC ↔ Printer ↔ Server

They all use private IPs and stay in the same LAN (same subnet).

✔️ Switch decisions are based on:

MAC Address Table

🟥 2. What is a Router?

A router operates at:

- Layer 3 (Network Layer) – IP addresses

✔️ What it does

- Connects different networks

- Connects LAN to WAN (Internet)

- Finds best path using routing protocols

- Performs NAT, firewall functions (in many devices)

✔️ Example

Connecting company LAN to Internet:

LAN ↔ Router ↔ Internet

✔️ Router decisions are based on:

Routing Table (IP address)

🟦 3. Super Simple Difference

| Switch | Router |
| --- | --- |
| Works with MAC addresses | Works with IP addresses |
| Operates on Layer 2 | Operates on Layer 3 |
| Used for LAN communication | Used for LAN ↔ WAN communication |
| Sends frames | Sends packets |
| Faster for local traffic | Slower but more intelligent |

🟩 4. Real-Life Example

Inside an office:

- All PCs connected to switch

- Switching based on MAC

To reach Internet:

- Traffic goes to router

- Router does NAT, routing

⭐ 5. Very Easy Memory Trick

✔️ Switch → “Connects devices inside a building”

✔️ Router → “Connects networks or buildings to each other”

🧩 6. Interview Line (Perfect Answer)

“A switch forwards frames based on MAC addresses (Layer 2).
A router forwards packets based on IP addresses (Layer 3).
Switch connects hosts in the same network; router connects different networks.”